Version ：GV-PF10S1-GG20170216

Shanghai Goodview Electronics Technology Co. Ltd.

Android

10inches LCD multimedia Display

Model:PF10S1

Revision History

| Part number | version | Revise Date | Prepared by | Checked by | approval | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1246001 | GV-PF10S1-GG20170216 | 2017-2-16 |  |  |  |  |

Note：All right reserved by Goodview.Please contact with us for the latest updates.

Product Specification

| Display Panel |  |  |  |  |
| --- | --- | --- | --- | --- |
| Panel |  | 10.1inches LED back-light/model:M101WXBI45-AA-01 |  |  |
| Active Area(mm) |  | 216.96 (W) ×135.60 (H) mm |  |  |
| Aspect Ratio |  | 16:10 |  |  |
| Resolution |  | 1280×800 |  |  |
| Brightness |  | 280/350 cd/m2 |  |  |
| Contrast Ratio |  | 800:1 |  |  |
| Viewing Angle |  | All angles ( R/L 89 (Min.), U/D 89 (Min.)) |  |  |
| Color |  | 16.7Mcolors |  |  |
| Color temperature |  | 5500K |  |  |
| Backlight lamp life |  | 30000 hours |  |  |
| Response time |  | 6ms |  |  |
| System |  |  |  |  |
| Main board model |  | SWH5128 |  |  |
| CPU |  | Rockchip RK3126 |  |  |
| GPU |  | ARM Mali-400MP2 GPU |  |  |
| Memory |  | DDR3 512Mbyte, |  |  |
| Storage |  | 8GB eMMC |  |  |
| Operation system |  | Android5.1 |  |  |
| Touch |  |  |  |  |
| Model |  |  |  | / |
| Touch type |  |  |  | / |
| Touch Points |  |  |  | / |
| Interpolation resolution |  |  |  | / |
| Touch accuracy |  |  |  | / |
| Response time |  |  |  | / |
| Enter mode |  |  |  | / |
| Smallest touch objects | Single |  |  | / |
|  | Multi-point |  |  | / |
| ports |  |  |  |  |
| Signal port |  | / |  |  |
| Network ports |  | LAN*1（10/100MEthernet），wifi (802.11n)、 blue-tooth(4.0) |  |  |
| Storage extension |  | 32GB（max） |  |  |
| Other |  | USB(2.0)*1、RS232*1（3.5mm jack）、POE(24V) |  |  |
| multimedia |  |  |  |  |
| Media file formats |  |  | video：MOV, MP4, AVI, MPG, 3PG |  |
|  |  |  | audio：MP3 |  |
|  |  |  | picture：JPEG、BMP、PNG |  |
|  |  |  | text：txt |  |
| Play mode |  |  | Landscape, vertical screen, full screen, and split screen |  |
| Program transmission way |  |  | USB, LAN, Wan transport |  |
| Other parameters |  |  |  |  |
| Product Size(mm) |  | 239.9（H）×169.2（V）×26.8（D） |  |  |
| Package Size(mm） |  | 333（H）×278（V）×75（D） |  |  |
| VESA（mm） |  | 50*50 |  |  |
| Screen hanging mode |  | Landscape, portrait |  |  |
| Menu language |  | Chinese, English |  |  |
| Machine color |  | Black |  |  |
| Weight |  | net：0.5KG gross：2.0KG |  |  |
| Power Consumption |  | 12W(Max) |  |  |
| Speaker |  | stereo 1.5W x8Ω |  |  |
| Watchdog timer |  | / |  |  |
| Timing function |  | multiple time periods programmable power on/off |  |  |
| Management software |  | GTV |  |  |
| Power supply |  | DC 12V |  |  |
| Working temperature |  | 0〜40℃ |  |  |
| Working humidity |  | 10% 〜 80%RH |  |  |
| Storage environment |  | - 10 ~ 50 ℃ temperature/ 10% ~ 90% RH humidity |  |  |
| Accessories |  | Warranty card * 1,Remote control * 1, certificate * 1, 12 v power adapter * 1,Network line* 1, safety and environmental considerations * 1,National standard power cord * 1,AAA battery * 2 |  |  |
| Certificates |  | CCC |  |  |

Schematic Drawing

Product Images

Packaging

Note：All right reserved by Goodview.